May 31, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-1

8.04 MHz Clock on Controller Board Will Not Oscillate

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

May 31, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-2

D654 Destroyed When a Linear is Connected

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

June 1, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-3

Blue Book Correction - Alignment

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

June 13, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-4

Receiver IF Breaks Into Oscillation After Transmission

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

June 13, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-5

Low or No Output From RF Board

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

June 13, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-6

S-Meter Rises Over Half Scale When IF Shift Control Is

Rotated

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

June 17, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-7

Blue Book Schematic Correction

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

June 17, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-8

Blue Book Corrections

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

July 25, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-9

RF Circuit Board Schematic Correction

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

August 29, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-10

Blue Book Audio Circuit Board Schematic Correction

C915

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

October 31, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-11

Blue Book Corrections - Power Amplifier Schematic

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

November 30, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-12

Distortion In Audio

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

December 19, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-13

Output Signal Distorted

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

December 19, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-14

Low Transmitter IF Signal to IF Amplifier

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

December 19, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-15

IF Amplifier Breaking into Oscillation

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

December 19, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-16

Dynamic Range of Audio Amplifier Compressed

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

December 19, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-17

BFO Drifting

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

December 19, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-18

Narrow CW Transmit Offset

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

December 19, 1983

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-19

Low Output from High VCO

+ + + + Information not yet available + + + +

----------------------------------------------------------------------------------------------

February 29, 1984

HW-5400                                                 Bulletin

No:

HF Transceiver                                          HW-5400-20

BFO And RIT Oscillators Have Insufficient Range;

RF and IF Performance Improvements

The following changes give greater tuning range to the BFO and RIT

oscillator circuits and improve performance of the RF and IF stages.

BFO Circuit

BFO Circuit Board:

Change C811 to 220 pF capacitor [PN 20-120].  This

capacitor may have been changed to a 180 pF [PN 21-746] per

Bulletin HW-5400-17, or it may still be the original 120 pF

[PN20-183].

RIT Circuit

IF Circuit Board:

Change R1145 from 1800 ohms to 2200 ohms [PN 6-222-12].

Change R1151 from 1000 ohms to 1500 ohms [PN 6-152-12].

Controller circuit board:

Change C703 and C704 from 330 pF to 220 pF [PN 20-120].

Change C720 back to a 3.2 - 18 pF [PN 31-71] if it has

been previously changed, per Bulletin HW-5400-18.

RF Circuits

RF circuit board:

Install a ferrite bead [PN 475-16] on the base lead of

Q401 and Q402.

IF circuit board:

Install a ferrite bead [PN 475-16] on the base lead of

Q1104 and Q1105.

PA circuit board:

Install a .01 uF capacitor [PN 21-176] between ground and

the lead of R1207 nearest T1202.

----------------------------------------------------------------------------------------------

March 9, 1984

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-21

Modification Summary

Below is a summary of all HW-5400 modifications, with applicable Bulletin

Nbrs.  They should be installed in each unit serviced; however, some may

already be present.

Audio Circuit Board

-- C908, remove and replace with a jumper wire. [HW-5400-12]

-- C915, remove and discard. [HW-5400-16]

-- C929, change from .1 uF to 330 pF [PN 21-722]. [HW-5400-14]

-- R913, change from 33 Kohms to 68 Kohms [PN 6-683-12]

[HW-5400-16]

High VCO Assembly

-- R367, change from 100 to 33 ohms [PN 6-330-12]. [HW-5400-19]

BFO Circuit Board

-- C811, change from 120 pF to 220 pF [PN 20-120]. [HW-5400-20]

-- C817, change from 7.7 pF to 27 pF [PN 21-6]. [HW-5400-17]

-- C819, change from 160 pF to .01 uF [PN 21-761]. [HW-5400-17]

-- C821, change from 7.7 pF to 27 pF [PN 21-6]. [HW-5400-17

-- L804, change from 22 uH toroid to 10 uH toroid [PN 45-57].

[HW-5400-17]

ALC Circuit Board

-- D654, change from a 1N5234B [PN 56-58] to a 1N4751 [PN 56-64].

[HW-5400-2]

Controller Circuit Board

-- C702, should remain the original value [3.2-18 pF] [PN 31-71].

If it was replaced per Bulletin HW-5400-18, reinstall a [PN 31-71].

-- C703, change from 330 pF to 220 pF [PN 20-120]. [HW-5400-20]

-- C704, change from 330 pF to 220 pF [PN 20-120], [HW-5400-20]

-- C701, add a second diode [PN 56-648] * in parallel [piggyback].

[HW-5400-18]

-- Q701, change from a MPS6521 to a 2N2369 [PN 417-154].

[HW-5400-1]

-- Q702, change from a MPS6521 to a 2N2369 [PN 417-154].

[HW-5400-1]

IF Circuit Board

-- R1145, change from 1800 ohms to 2200 ohms [PN 6-222-12].

[HW-5400-20]

-- R1151, change from 1000 ohms to 1500 ohms [PN 6-152-12].

[HW-5400-20]

-- Q1104, install a ferrite bead [PN 475-16] on the base lead.

[HW-5400-20]

-- Q1105, install a ferrite bead [PN 475-16] on the base lead.

[HW-5400-20]

Power Amplifier Circuit Board

-- C1206, install a .01 uF capacitor [PN 21-176] between ground

and the lead of R1207 nearest transformer T1202.  [HW-5400-20]

RF Circuit Board

-- Q401, install a ferrite bead [PN 475-16] on the base lead.

[HW-5400-20]

-- Q402, install a ferrite bead [PN 475-16] on the base lead.

[HW-5400-20]

----------------------------------------------------------------------------------------------

May 31, 1984

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-22

Blue Book Correction

Dual Banana Plug Part Numbers

In the HW-5400 Service Data manual [Blue Book], pages 166 and 169, change

the part number of the dual-banana plug from 438-47 to 438-30.

----------------------------------------------------------------------------------------------

July 30, 1984

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-23

Wrong Size Pressed-in Spacers On Chassis

Some chassis for this unit have incorrect size pressed-in spacers

installed.  The incorrect size spacers are 1/8" too short and are the

wrong thread size; 4-40 instead of 6-32.  However, most of the chassis

have the correct size spacers and those chassis which have incorrect size

spacers, only two or three spacers may be the wrong size.

The pressed-in spacers are used, in most cases, to mount a circuit board

onto the chassis.  If the incorrect size spacers are tight in the

chassis, you may secure the circuit board at these spacers by installing

the following hardware.

-- Refer to the drawing at the right and install a 4-40 x 3/4"

screw [PN250-312] from the opposite side of chassis through the

spacer. Tighten this screw into the spacer.

-- Install a 4-40 nut [PN 252-2] on the end of the 4-4 x 3/4"

screw and tighten it down onto the spacer.

-- Mount the circuit board with another 4-40 nut [PN 252-2].

Install this hardware as needed.

----------------------------------------------------------------------------------------------

August 10, 1984

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-24

PN 56-648 Diode No Longer Available

The MV109 varator diodes at D101, D301, D351, D701 and D701b are changed:

From PN 56-648 to 56-674.

This new diode is an MV209 and has a different case style.  Install it as

at the right. [[Note: shows mounting the flat side with pc board marked

'band']].  Use this new diode when a replacement is needed.

----------------------------------------------------------------------------------------------

August 10, 1984

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-25

Output On 30 And 40 Meters Too High

To reduce the output on the 30 meter and 40 meter bands, install a trap

on the RF circuit board.

Parts Needed

Qty     Description             Heath PN

1    470 ohm resistor         6-471-12

1    15 pF ceramic capacitor 21-707

1    22 uH choke             45-631

Installation

-- Connect the 470 ohm resistor, the 15 pF capacitor and the 22uH choke

in series as shown at the right.  [[Shows one end of 22uH choke

mounted vertically into hole marked 'A' , the 15pF capacitor soldered

to the free end of the choke.  The other end of the cap soldered to

the 470 ohm resistor, which in turn is mounted vertically into hole

on PCB marked 'B'.]]

-- On the RF board, install the RLC network [[Above]] in the two unused

circuit board holes near D404.  These holes are marker A and B on the

x-ray view at the right.  Install the choke in hole A.

----------------------------------------------------------------------------------------------

August 17, 1984

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-26

Improved CW Keying

On the RF circuit board, make the following circuit changes.

Parts needed

QTY     Description             Heath PN

2    1200 ohm resistor        6-122-12

1    Diode                   56-56

1    10 uF capacitor         25-864

1    .68 uF capacitor        25-922

Procedure

-- On the RF circuit board, remove the 2200 ohm resistor at R415.

-- Remove the 10 uF capacitor at C424.

-- Change the 4.7 uF electrolytic capacitor at C418 to a .68 uF

electrolytic capacitor [PN 25-922]

-- Refer to the drawing at the right and connect the two 1200 ohm

resistors [PN 6-122-12] and the [PN 56-56] in series.  The banded end

of the diode connects to the end of a 1200 ohm resistor.

[[[diode - resistor - resistor]]]

-- Install the resistor - diode combination at R415 on the board.  The

free end of the 1200 ohm resistor connects to the base of Q403.

The anode of the diode connects to the collector of Q412.

-- Connect the 10 uF capacitor [PN 25-864] from the junction of the 1200

ohm resistors to the ground end of R414.

Install these changes only when needed.

----------------------------------------------------------------------------------------------

August 24, 1984

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-27

Unable To Adjust R803 FOr 8.8307 Mhz

On the BFO board, you may find that when you adjust R837 fully clockwise,

the 8.8307 Mhz frequency is still 100 to 200 Hz too high.  In this case,

change:

R841 from a 47 kilohm resistor to a 10 kilohm resistor [PN6-103-12].

After this resistor change is made, repeat the BFO alignment from the

beginning.

Make this change only when needed.

----------------------------------------------------------------------------------------------

September 21, 1984

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-28

Color Dot Missing On Band Switch SW401A/B/C

In Factory Service, some of the wafer switches [PN 63-1386] at SW401A/B/C

were found installed backwards because the color dot used as an

orientation mark on each switch was missing.

To correctly install a switch with no color dot, use the circled number

as the orientation mark.  This circled number is on the same side of the

switch as the color dot would normally be.

----------------------------------------------------------------------------------------------

January 31, 1985

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-29

Intermittent Lockout Of PLL

The [PN 52-193] transformers used at T101, T102, T103 and T104 do not

have enough adjustment range to enable absolute peaking of the RF level

specified in the synthersizer alignment instructions.  Some of these

transformer's cores will be all the way up or all the way down and do not

reach the peaking point.  Since the RF levels are not peaked, this may

cause intermittent PLL problems.

If the unit you're servicing has an intermittent PLL problem, change:

T101, T102, T103 and T104 from a [PN 52-193] transformer to a [PN52-182] transformer.

Change these transformers only when needed.

Future HW-5400 Transceiver kits will have the [PN 52-182] transformers.

T105 will remain a [PN 52-193] transformer.

----------------------------------------------------------------------------------------------

January 31, 1985

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-30

IF Oscillation

On the IF board, IF oscillation problems are being created by the [PN

417-172] transistors used Q1104, Q1105 and Q1106.  Each transistor causes

particular symptoms when it oscillates.  The symptoms are:

Q1104 - Causes upscale meter deflections.

Q1105 - Meter jumps full scale in the transition period between

transmit and receive.  Also, the AGC is too slow.

Q1106 - Meter jumps upscale when temperature varies slightly.

To correct, change:

Q1104, Q1105 and Q1106 from [PN 417-172] transistors to [PN417-293]

transistors.

Install these transistors in the units received for service.

----------------------------------------------------------------------------------------------

January 31, 1985

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-31

Won't Meet Overall Receiver Gain Specs

On the audio board, the previous change of R994 and R995 from a 1000 ohm

resistor to a 47 ohm resistor decreased the audio gain, and now the

Transceiver will not meet overall receiver gain specs.  To ensure the

Transceiver meets specs, change:

R994 and R995 from a 47 ohm resistor to a 1000 ohm resistor

[PN 6-102-12].

Install the 1000 ohm resistors in all units received for service.

The bulletin supercedes HW-5400-15 in the 1983 book.

----------------------------------------------------------------------------------------------

March 15, 1985

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-32

Screws Break Off In Pressed-in Spacers On Chassis

On the main and rear chassis, screws may bind in the pressed-in spacers,

causing the screw to break.  This problem has occurred only on the

copper-colored chassis.  To correct the problem, drill out the broken

screw.  If the spacer with the broken screw is used to mount a circuit

board, remove all the board-mounting hardware and move the board aside.

Drill out the broken screw from the bottom side of the spacer.  Select a

drill bit that fits inside the spacer so that you don't drill the hole

too large to retap for a 6-32 machine screw.  As you drill out the screw,

be sure to keep the metal particles out of the unit.  Retap with a 6-32

tap.

An insert [PN 591-4676] with two self-tapping screws attached is being

put into the present stock of kits.  The customer is instructed to use

the self-tapping screws to retap all threaded studs before starting

chassis assembly.  Future production of kits will have a chassis with

properly threaded spacers.

----------------------------------------------------------------------------------------------

March 22, 1985

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-33

Transmitter Self-Oscillates In Tune Mode With CW Gain

Control FCCW

On the IF board, install a 4700 ohm resistor [PN 6-472-12] from the base

of Q1102 to ground.  This can be done on the top of the board by

connecting one resistor lead to the base at R1107 and the other lead to

the grounded end of R1103.

This additional 4700 ohm resistor ensures that Q1101 and Q1102 turn off

completely during transmit.

----------------------------------------------------------------------------------------------

March 22, 1985

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-34

Little Or No ALC Cutback In Transmit Power When Operating

Into 3 To 1 Impedance Mismatch [150 Ohm Load]

The ALC cutback operation is tested by connecting the transmitter to a

150 ohm load.  The 3 to 1 mismatch should cause the ALC circuitry to

reduce the transmitter output power about 30%.  To improve the ALC

cutback with high mismatches, on the ALC board:

Remove the 27 kilohm resistor at R659.

----------------------------------------------------------------------------------------------

May 31, 1985

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-35

Diode Change At D903 And D904

On the audio board, the [PN 56-656] diodes at D903 and D904 are changed

to [PN 56-56] diodes.  The #56-656 diodes are obsolete.

----------------------------------------------------------------------------------------------

May 31, 1985

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-36

Schematic Correction - Blue Book

On the BFO schematic in the HW-5400 Blue Book, P801-2 should go to

P106-1. P802-2 should go to P905-9.  Mark this correction on your yellow

schematic.

----------------------------------------------------------------------------------------------

May 31, 1989

HW-5400                                         Bulletin No:

HF SSB Transceiver                              HW-5400-37

1983 TEB Book Correction

In the 1983 TEB book, TEB HW-5400-6, page 143, last paragraph, change

"P918" to read "P905, pin 4".

----------------------------------------------------------------------------------------------

That's all I have listed for the HW-5400 (1983-89).  Enjoy!

73 de Joe  W7LPF/4  [NNN0KUU]

QWCA - SOWP - NCVA - FISTS - RCC