MiniUART IP Core
Specification

#### Author: Philippe Carton

Philippe.carton2@libertysurf.fr

Rev. 0.1

July j, 2026

##### This page has been intentionally left blank

##### Revision History

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 28/12/01 | Philippe Carton | First Draft |
| 0.2 | 5/3/02 | Philippe Carton | Detailed Baudrate tolerance section |

Contents

Introduction	………………………………………………	1

IO ports 	………………………………………………	2

Clocks 	………………………………………………	Error: Reference source not found

Registers 	………………………………………………	4

Operation 	………………………………………………	6

Architecture 	………………………………………………	9

# 1

Introduction

This UART (Universal Asynchronous Receiver Transmitter) is designed to make an interface between a RS232 line and a wishbone bus, or a microcontroller, or an IP core.

It works fine connected to the serial port of a PC for data exchange with custom electronic.

It was built in the perspective to be very small, but efficient. It had to fit in a small FPGA.

It is not suited to interface a modem as there is no control handshaking (CTS/RTS).

It integrate two separate clocks, one for wishbone bus, the other for bitstream generation. This has the advantage to let the user bring his own desired frequency for the baudrate.

Support:

- WISHBONE interface in 8-bit data bus

- Two clock: one for wishbone interface, one for RS232 bitstream generation

- Baudrate divisor from 1 to 65536 (generic parameter set at integration time)

Does not support:

- FIFO input/output

- Control handshaking

# 2

IO ports

## 2.1 WISHBONE interface signals

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| WB_CLK_I | 1 | Input | Block’s clock input |
| WB_RST_I | 1 | Input | Asynchronous Reset |
| WB_ADDR_I | 2 | Input | Used for register selection |
| WB_DAT_I | 8 | Input | Data input |
| WB_DAT_O | 8 | Output | Data output |
| WB_WE_I | 1 | Input | Write or read cycle selection |
| WB_STB_I | 1 | Input | Specifies transfer cycle |
| WB_ACK_O | 1 | Output | Acknowledge of a transfer |

## 2.2 Other internal signals

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| IntTx_O | 1 | Output | Transmit Interrupt |
| IntRx_O | 1 | Output | Receive Interrupt |
| BR_CLK_I | 1 | Output | Clock for serialisation/unserialisation |

## 2.3 External (off-chip) connections

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| TxD_PAD_O | 1 | Output | The serial output signal |
| RxD_PAD_I | 1 | Input | The serial input signal |

# 3

Clocks

Clocks table:

| Name | Source | Rates (MHz) |  |  | Remarks | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Max | Min | Resolution |  |  |
| WB_CLK_I | Wishbone bus | Limited by target host |  |  | None | Wishbone clock |
| BR_CLK_I | User |  |  |  | None | Baudrate clock |

# 4

Registers

## 4.1 Registers list

| Name | Address | Width | Access | Description |
| --- | --- | --- | --- | --- |
| Receive buffer | 0 | 8 | R | Contain byte received |
| Transmit buffer | 0 | 8 | W | Contain byte to transmit |
| Status | 1 | 8 | R | Receive buffer full / Transmitter busy |
| Reserved | 2 | 8 |  |  |
| Reserved | 3 | 8 |  |  |

## 4.2 Status register

| Bit # | Access | Description |
| --- | --- | --- |
| 0 | R | Transmitter buffer state = IntTx_O pin ‘0’ – Busy. Can’t accept incoming byte ‘1’ – Accept a byte to transmit |
| 1 | R | Receiver buffer state = IntRx_O pin ‘0’ – Buffer empty ‘1’ – Buffer contain a received byte |

Reset Value: XXXXXX01b

# 5

Operation

The UART Operation is very basic :

Upon a write to the data input bus WB_DAT_I, the core will automatically serialise and emit the byte on the TxD_PAD_O. It will hold IntTx_O low as long as it cannot accept an incoming byte. Therefore a rising edge on IntTx_O can trigger the interrupt line of a microcontroller to emit another byte.

Upon reception of a bitstream on RxD_PAD_O, the core will unserialise the information and assert IntRx_O pin. This announce that the received byte can be read on the data output bus WB_DAT_O. As soon as the byte is read, IntRx_O is negated.

## Wishbone bus

The core is 8 bit wishbone compatible. It doesn’t use the WB_CYC_I pin as it will never insert wait states.

## Initialization

The core doesn’t need to be reset, as it is ready to use upon power on.

However, a synchronous assertion of WB_RST_I will abort any pending transmit / receive and will set the core in idle state.

## Baudrate divisor

A generic VHDL parameter allow the user to introduce a divisor between BR_CLK_I clock and the bitstream frequency. In addition, the core insert a 4 divisor for sampling purpose for the receiver.

Thus:

Baudrate = Freq(BR_CLK_I) / BRDIVISOR / 4

Where BRDIVISOR is the generic parameter.

To instanciate the component in a VHDL unit, write:

U1 : MiniUART

generic map (BRDIVISOR => 103)

port map (clk, rst, adr, ...);

for a division of 103.

| Below are given some divisor values for common baudrates: BRDIVISOR | Baudrate |
| --- | --- |
| 2080 | 1200 |
| 1040 | 2400 |
| 520 | 4800 |
| 260 | 9600 |
| 130 | 19200 |
| 65 | 38400 |

BR_CLK_I at 10MHz

| BRDIVISOR | Baudrate |
| --- | --- |
| 192 | 2400 |
| 96 | 4800 |
| 48 | 9600 |
| 32 | 14400 |
| 24 | 19200 |
| 16 | 28800 |
| 8 | 57600 |
| 4 | 115200 |
| 2 | 230400 |
| 1 | 460800 |

BR_CLK_I at 1.8432MHz

## Baudrate tolerance

The receiver will accept a light variation between the expected baudrate and the effective bitstream baudrate that is:

| Min freq | Nominal Baudrate -2.60% |
| --- | --- |
| Max freq | Nominal Baudrate +5.90% |

Equivalent in period :

| Min period | Nominal period -5.90% |
| --- | --- |
| Max period | Nominal period +2.60% |

Going beyond this limits, the receiver will be unable to deserialise correctly.

# 6

Architecture

The block diagram of the core is given:

Figure 1 - Block Diagram of the UART core