Note for implementing the MiniUART design in Foundation

The MiniUART IPCore can be integrated in a schematic design as a black box.

- Create or open a schematic design. If you create a new design, select the schematic radio button.

- Copy the four vhdl file: miniuart.vhd, txunit.vhd, rxunit.vhd and utils.vhd in the main directory of the project.

- In the Project manager, click Project->Add source File(s). In the Add document window, select the four vhdl files added in 2.

- For each vhdl file, open it with the HDL editor and click Synthesis->Check syntax

- Launch the HDL editor. Select existing document radio button. Choose miniuart.vhd in the file dialog box. Click Synthesis->Synthesize. This will have the effect to synthesize the miniuart with its corresponding dependencies. You should only see a warning : “The net '/UART/WB_STB_I' is a feed through net.  (FPGA-CHECK-9)”

- Click Project->Create macro. This will create a macro for the miniuart. You can see it in the Synthesis tab of the Project Manager. You can also see it as a SC symbol in the schematic editor. You can now place it in a schematic and add wires and pads. Then you can implement your design like you used to do with the traditional Implementation push button.